# HayesX-250

# Ultralight Aircraft

# Maintenance Manual

目录

PowerHub SkyLo V1	1

超轻型飞行器维护手册	1

Ultralight Aircraft Maintenance Manual	1

第 1 章 总则	7

Chapter 1 General	7

  - 手册目的	7

  - Purpose	7

  - 维护基本原则	7

  - Basic Maintenance Principles	7

第 2 章 维护安全要求	8

Chapter 2 Maintenance Safety	8

  - 高压系统安全	8

  - High-Voltage System Safety	9

第 3 章 飞行前检查	10

Chapter 3 Pre-Flight Inspection	10

  - 机体结构	10

  - Airframe Structure	10

  - 碳纤维旋翼	11

  - Carbon-Fiber Rotors	11

第 4 章 电机维护	12

Chapter 4 Motor Maintenance	12

  - 日常检查	12

  - Routine Inspection	12

  - 500 小时定期检查	13

  - 500-Hour Scheduled Inspection	13

第 5 章 ECS 维护	14

Chapter 5 ECS Maintenance	15

  - 日常检查	15

  - Routine Inspection	15

  - 500 小时定期检查	15

  - 500-Hour Scheduled Inspection	16

第 6 章 碳纤维旋翼维护	17

Chapter 6 Carbon-Fiber Rotor Maintenance	17

  - 每次飞行检查	17

  - Inspection Before Each Flight	17

  - 500 小时定期检查	18

  - 500-Hour Scheduled Inspection	18

第 7 章 高压动力电池维护	19

Chapter 7 High-Voltage Propulsion Battery Maintenance	19

  - 飞行前检查	20

  - Pre-Flight Inspection	20

  - 400 循环定期检查	21

  - 400-Cycle Scheduled Inspection	21

  - 800 循环性能检查	22

  - 800-Cycle Performance Inspection	22

  - 电池循环定义	24

  - Battery Cycle Definition	24

第 8 章 电气线束维护	25

Chapter 8 Electrical Wiring Maintenance	26

  - 信号线束	26

  - Signal Wiring	26

  - 主动力线束	27

  - Main Power Wiring	27

第 9 章 飞控系统维护	28

Chapter 9 Flight Control System Maintenance	28

第 10 章 整机救援降落伞维护	29

Chapter 10 Whole-Aircraft Emergency Parachute Maintenance	29

第 11 章 座椅及乘员保护系统	30

Chapter 11 Seat and Occupant Protection System	30

第 12 章 定期维护计划	31

Chapter 12 Scheduled Maintenance Program	32

第 13 章 时寿件与强制更换件清单	33

Chapter 13 Life-Limited Parts and Mandatory Replacement Items	33

  - 总则	33

  - General	33

  - SkyLo V1 时寿件清单	34

  - SkyLo V1 Life-Limited Parts List	34

  - GBS 10 救援降落伞系统	34

  - GBS 10 Rescue Parachute System	34

  - GBS 10 容器检查	35

  - GBS 10 Container Inspection	35

  - GBS 10 热释动器	37

  - GBS 10 Pyroactuator	37

  - GBS 10 激活后的维护	38

  - Post-Activation Maintenance	38

  - 救援系统维护记录	39

  - Rescue System Maintenance Records	39

  - GPS 底座维护要求	41

  - GPS Mount Base Maintenance Requirements	41

  - 差分天线底座维护要求	42

  - Differential GNSS Antenna Mount Base Maintenance Requirements	43

第 14 章 长期存放	45

Chapter 14 Long-Term Storage	45

第 15 章 维护记录	46

Chapter 15 Maintenance Records	46

第 16 章 维护放行	47

Chapter 16 Return to Service	47

附录 A：主要维护间隔	48

Appendix A: Principal Maintenance Intervals	48

附录 B：当前构型参数	49

Appendix B: Current Configuration Data	49

第 1 章 总则 Chapter 1 General

## 手册目的

## Purpose

本手册规定 PowerHub SkyLo V1 超轻型飞行器的日常检查、飞行前检查、飞行后检查、定期维护、故障处置、部件更换以及长期存放维护要求。

This manual specifies the requirements for daily inspection, pre-flight inspection, post-flight inspection, scheduled maintenance, troubleshooting, component replacement, and long-term storage of the PowerHub SkyLo V1 ultralight aircraft.

本手册适用于飞行员、地面保障人员及经授权的维修人员。

This manual is applicable to pilots, ground support personnel, and authorized maintenance personnel.

## 维护基本原则

## Basic Maintenance Principles

SkyLo V1 采用模块化分布式电推进系统，主要维护对象包括：

The SkyLo V1 adopts a modular distributed electric propulsion system. The principal maintenance items include:

机体主承力结构 / Primary airframe structure碳纤维旋翼 / Carbon-fiber rotors

电机 / Motor ECS / ECS units

高压动力电池 / High-voltage battery packs

高压配电系统 / High-voltage power distribution system飞控系统 / Flight control system

信号及动力线束 / Signal and power wiring

整机救援降落伞系统 / Whole-aircraft emergency parachute system座椅及乘员保护系统 / Seat and occupant protection system

所有维护工作必须按照本手册、相关部件维护文件及经批准的技术变更文件执行。 All maintenance activities shall be performed in accordance with this manual, applicable component maintenance documentation, and approved technical change documentation.

# 第 2 章 维护安全要求

# Chapter 2 Maintenance Safety

## 高压系统安全

## High-Voltage System Safety

SkyLo V1 采用双路高压动力电池系统。

The SkyLo V1 is equipped with a dual high-voltage propulsion battery system.

维护高压系统前必须：

Before maintenance of the high-voltage system:

1.

关闭飞行器主电源；

Switch off the aircraft main power.

2.

断开高压动力电池；

Disconnect the high-voltage propulsion batteries.

3.

确认高压系统已经完成放电及隔离；

Verify that the high-voltage system has been discharged and electrically isolated.

使用适当的测量设备确认不存在危险电压； 4.

Verify the absence of hazardous voltage using appropriate test equipment.

5.

未经授权人员不得进行高压系统维修。

High-voltage system maintenance shall only be performed by authorized personnel.

# 第 3 章 飞行前检查

# Chapter 3 Pre-Flight Inspection

- 机体结构

## Airframe Structure

检查以下项目： Inspect the following:

主机身结构是否存在裂纹；

Check the primary fuselage structure for cracks.碳纤维结构是否存在分层、撞击损伤或明显划伤；

Check carbon-fiber structures for delamination, impact damage, or significant scratches.

主承力连接部位是否松动；

Check primary structural attachment points for looseness.

旋翼支臂是否存在变形；

Check rotor arms for deformation.折叠机构及锁止机构是否正常。

Check the folding and locking mechanisms for proper operation.发现影响结构完整性的异常情况时，禁止飞行。

If any condition affecting structural integrity is found, flight shall not be permitted.

## 碳纤维旋翼

- Carbon-Fiber Rotors

每次飞行前检查：

Before every flight, inspect:裂纹 / Cracks

缺口 / Nicks

分层 / Delamination变形 / Deformation

异物撞击痕迹 / Foreign-object impact marks

安装松动 / Installation looseness

旋翼与电机连接状态 / Rotor-to-motor attachment condition任何旋翼出现裂纹、明显分层、结构性缺口或变形时，不得继续使用。

A rotor shall not be used if cracks, significant delamination, structural damage, or deformation is detected.

# 第 4 章 电机维护

# Chapter 4 Motor Maintenance

## 日常检查

## Routine Inspection

飞行前检查电机：

Before flight, inspect each motor for:安装螺栓松动；

Loose mounting bolts.异常声音；

Abnormal noise.异常振动；

Abnormal vibration.转动阻力异常；

Abnormal rotational resistance.温度异常；

Abnormal temperature.

电源线及连接器损伤。

Damage to power cables or connectors.

## 500 小时定期检查

## 500-Hour Scheduled Inspection

### 电机规定定检间隔：500 飞行小时。

### Prescribed motor inspection interval: 500 flight hours.

达到 500 飞行小时后，应对全部电机进行定期检查。

At 500 flight hours, all motors shall undergo scheduled inspection.检查项目包括：

Inspection shall include:

1.

电机外观及壳体检查；

Visual inspection of motor housing and external condition.

2.

电机轴及轴承状态检查；

Inspection of motor shaft and bearing condition.

3.

电机安装接口检查；

Inspection of motor mounting interfaces.

4.

电机旋转阻力检查；

Check of rotational resistance.

data.

5.

电机振动及异常噪声检查；

Check for abnormal vibration and noise.

6.

电气连接及绝缘状态检查；

Inspection of electrical connections and insulation condition.

7.

电机运行温升检查；

Operational temperature-rise inspection.

8.

电机运行参数及历史数据检查。

Review of motor operating parameters and recorded operating

9.

如发现轴承异常、明显振动、过热、绝缘异常或结构损伤，应停止使用并按照维修程序处理。

If abnormal bearing condition, excessive vibration, overheating, insulation abnormality, or structural damage is detected, the motor shall be removed from service and maintained in accordance with the applicable maintenance procedure.

# 第 5 章 ECS 维护

# Chapter 5 ECS Maintenance

## 日常检查

## Routine Inspection

检查：

Inspect:

•

ECS 外观；

ECS external condition.电源连接；

Power connections.电机连接；

Motor connections.控制信号连接；

Control signal connections.散热状态；

Cooling condition.

过热、烧蚀或异常气味；

Evidence of overheating, burning, or abnormal odor.故障记录。

Fault records.

## 500 小时定期检查

## 500-Hour Scheduled Inspection

### ECS 规定定检间隔：500 飞行小时。

### Prescribed ECS inspection interval: 500 flight hours.

500 飞行小时定检包括：

The 500-hour inspection shall include:外观及结构检查；

Visual and structural inspection.功率连接器检查；

Power connector inspection.信号连接器检查；

Signal connector inspection.线束及端子检查；

Wiring and terminal inspection.散热系统检查；

Cooling system inspection.工作温度及温升检查；

Operating temperature and temperature-rise inspection.故障日志检查；

Fault-log inspection.电机控制响应检查；

Motor-control response inspection.电气参数检查。

Electrical parameter inspection.

发现过热、异常报警、功率输出异常、连接器烧蚀或其他异常情况时，应进一步进行专项检查。

If overheating, abnormal alarms, abnormal power output, connector burning, or other abnormalities are found, a detailed inspection shall be performed.

# 第 6 章 碳纤维旋翼维护

# Chapter 6 Carbon-Fiber Rotor Maintenance

## 每次飞行检查

## Inspection Before Each Flight

旋翼必须进行目视检查。

Each rotor shall receive a visual inspection.重点检查：

Particular attention shall be given to:桨叶前缘；

Leading edge.桨叶后缘； Trailing edge.桨尖；

Blade tip.桨根； Blade root.表面分层；

Surface delamination.裂纹；

Cracks.

撞击损伤； Impact damage.

安装孔及连接区域。

Mounting holes and attachment areas.

## 500 小时定期检查

## 500-Hour Scheduled Inspection

### 碳纤维旋翼规定定检间隔：500 飞行小时。

### Prescribed carbon-fiber rotor inspection interval: 500 flight hours.

500 飞行小时定检应包括：

The 500-hour inspection shall include:旋翼整体外观检查；

Overall visual inspection.桨叶裂纹检查；

Blade crack inspection.碳纤维分层检查；

Carbon-fiber delamination inspection.桨根连接区域检查；

Blade-root attachment inspection.安装孔及紧固件检查；

Inspection of mounting holes and fasteners.旋翼平衡状态检查；

Rotor balance inspection.必要时进行无损检测。

Non-destructive inspection when required.

任何影响结构完整性或动平衡状态的缺陷，均应按照维修程序进行处理。 Any defect affecting structural integrity or dynamic balance shall be handled in accordance with the applicable maintenance procedure.

第 7 章 高压动力电池维护 Chapter 7 High-Voltage

Propulsion Battery Maintenance

SkyLo V1 采用双路高压动力电池系统，两组电池分别为上下旋翼动力系统供电。

The SkyLo V1 uses a dual high-voltage propulsion battery system. The two battery packs independently supply power to the upper and lower rotor propulsion systems.

动力电源路径 / Propulsion Power Path：电池 A → ECS → 上层旋翼

Battery A → ECS → Upper Rotors电池 B → ECS → 下层旋翼

Battery B → ECS → Lower Rotors

两组动力电池具有独立的供电路径及保护功能。

The two propulsion battery packs have independent power supply paths and protection functions.

## 飞行前检查

## Pre-Flight Inspection

每次飞行前检查：

Before each flight, inspect:

电池外观 / Battery enclosure

电池连接器 / Battery connectors电池电压 / Battery voltage

电池 SOC / State of charge

电池温度 / Battery temperature

电池安装及锁止状态 / Battery installation and locking condition出现以下情况时不得继续使用：

The battery shall not be used if any of the following conditions are observed:

外壳明显变形 / Significant enclosure deformation

鼓包 / Swelling

裂纹或破损 / Cracks or physical damage电解液泄漏 / Electrolyte leakage

异常发热 / Abnormal heating

明显容量衰减 / Significant capacity degradation

## 400 循环定期检查

## 400-Cycle Scheduled Inspection

### 动力电池规定定期检查间隔：每 400 个完整充放电循环。

### Prescribed propulsion battery inspection interval: every 400 complete charge-discharge cycles.

达到 400 个完整充放电循环后，应对动力电池进行定期检查。 Upon reaching 400 complete charge-discharge cycles, the

propulsion battery shall undergo a scheduled inspection.检查内容包括：

The inspection shall include:

电池外观及结构完整性检查；

Visual and structural integrity inspection.电池连接器及高压接口检查；

Inspection of battery connectors and high-voltage interfaces. BMS 运行状态及故障记录检查；

BMS operational status and fault-log review.单体电压一致性检查；

Cell voltage consistency inspection.电池温度及温度一致性检查；

Battery temperature and temperature consistency inspection.充放电数据检查；

Review of charge and discharge data.电池内阻或等效内阻参数检查；

Battery internal resistance or equivalent resistance assessment.电池容量状态评估。

Battery capacity condition assessment.检查结果应记录于动力电池维护记录中。

The inspection results shall be recorded in the propulsion

battery maintenance records.

## 800 循环性能检查

## 800-Cycle Performance Inspection

### 动力电池达到 800 个完整充放电循环后，应进行专项性能检查。

### A dedicated battery performance inspection shall be performed after 800 complete charge-discharge cycles.

800 循环性能检查的目的，是评估动力电池是否仍满足航空器正常运行所需的容量、功率输出及安全性能要求。

The purpose of the 800-cycle performance inspection is to determine whether the propulsion battery continues to meet the required capacity, power output, and safety performance requirements for aircraft operation.

性能检查包括：

The performance inspection shall include:

实际容量测试 / Actual capacity test

最大持续放电能力检查 / Maximum continuous discharge capability

电压一致性检查 / Cell voltage consistency内阻检查 / Internal resistance assessment

温升特性检查 / Temperature-rise characteristics BMS 保护功能检查 / BMS protection-function check充电功能检查 / Charging-function check

放电功能检查 / Discharge-function check故障记录分析 / Fault-history analysis

根据性能检查结果，对电池进行以下分类：

Based on the inspection results, the battery shall be classified as:

### A 类：继续使用 / Category A — Continue in Service

电池容量、功率输出、内阻、温升及 BMS 状态满足设计要求。

Battery capacity, power output, resistance, temperature rise, and BMS condition meet the applicable design requirements.

### B 类：限制使用 / Category B — Restricted Service

电池仍具备使用条件，但性能出现明显衰减，应根据技术人员评估降低使用范围或缩短后续检查间隔。

The battery remains serviceable but shows significant degradation. Its operating limitations or subsequent inspection interval shall be adjusted based on technical assessment.

### C 类：停止使用 / Category C — Remove from Service

电池存在安全风险，或者性能已经不能满足航空器运行要求。

The battery presents a safety risk or no longer meets the requirements for aircraft operation.

## 电池循环定义

## Battery Cycle Definition

为保证维护记录的一致性，SkyLo V1 将一个完整充放电循环定义为：

For maintenance-record consistency, one complete battery cycle for the SkyLo V1 is defined as:

### 累计放电量达到电池额定容量 100%的使用过程，计为 1 个完整循环。

### A cumulative discharge equivalent to 100% of the rated battery capacity shall be counted as one complete cycle.

例如：

For example:

一次从 100% SOC 使用至 50% SOC，再充电至 100%，计为约 0.5 个循环。

Using the battery from 100% SOC to 50% SOC and then recharging to 100% corresponds to approximately 0.5 cycle.

两次上述使用过程累计计为约 1 个完整循环。

Two such operating cycles correspond to approximately one complete cycle.

# 第 8 章 电气线束维护

Chapter 8 Electrical Wiring

Maintenance

## 信号线束

## Signal Wiring

SkyLo V1 信号控制线采用 PTFE 航空镀银屏蔽线。

SkyLo V1 signal-control wiring uses PTFE-insulated, silver-plated, shielded aerospace wire.

主要规格：

Principal specifications:

规格：4 × 0.35 mm² / 4 × 0.35 mm²外径：4.1 mm / OD: 4.1 mm

温度范围：−90 ℃ ～ +260 ℃ / Temperature range: −90°C to

+260°C

•

检查： Inspect:

绝缘层； Insulation.屏蔽层； Shielding. 连接器； Connectors.

固定点； Attachment points.磨损及挤压损伤。

Chafing and crushing damage.

## 主动力线束

## Main Power Wiring

主动力线采用 8AWG / 约 6 mm²硅胶线缆。

Main power wiring uses 8 AWG / approximately 6 mm² silicone-insulated cable.

主要规格：

Principal specifications:

规格：8AWG / 6 mm²持续载流能力：50 A

Continuous current rating: 50 A温度范围：−60 ℃ ～ +200 ℃

Temperature range: −60°C to +200°C应重点检查：

Particular attention shall be given to:绝缘层烧蚀；

Insulation burning or thermal damage.线缆磨损；

Cable abrasion.

接线端子； Terminals.

连接器温升痕迹；

Evidence of connector overheating.线束固定状态。

Wiring attachment condition.

# 第 9 章 飞控系统维护

# Chapter 9 Flight Control System Maintenance

当前飞控软件构型：

Current flight-control software configuration:

### V9 81.11.0

飞控软件版本必须纳入整机构型管理。

The flight-control software version shall be controlled as part of the aircraft configuration management system.

维护人员不得未经授权修改飞控参数或安装未经批准的软件版本。 Maintenance personnel shall not modify flight-control parameters or install unapproved software versions without authorization.

每次重大维护后应进行：

After major maintenance, the following shall be performed:

飞控自检 / Flight-control self-test传感器检查 / Sensor check

电机控制检查 / Motor-control check

地面动力系统检查 / Ground propulsion-system check

必要的功能飞行验证 / Functional flight verification when required

•

第 10 章 整机救援降落伞维护 Chapter 10 Whole-Aircraft Emergency Parachute Maintenance

救援降落伞系统属于关键安全设备。

The whole-aircraft emergency parachute system is a critical safety system.

维护要求应同时遵循降落伞系统制造商的维护文件。

Maintenance shall also comply with the applicable parachute-system manufacturer's maintenance documentation.

检查： Inspect:

降落伞系统有效期； Parachute system service life.

发射/释放装置； Deployment/release mechanism.操作手柄；

Activation handle.连接带；

Suspension/attachment straps.固定结构；

Mounting structure.系统保险状态。 Safety/arming status.

未经授权不得拆解、重新折叠或改装救援降落伞系统。

Unauthorized disassembly, repacking, or modification of the emergency parachute system is prohibited.

第 11 章 座椅及乘员保护系统 Chapter 11 Seat and Occupant Protection System

SkyLo V1 采用 T300/T700 全碳纤维半包覆式座椅。

The SkyLo V1 uses a T300/T700 all-carbon-fiber semi-enclosed seat.座椅两侧连接于主机身结构侧梁。

The seat is attached on both sides to the main fuselage structural side beams.

最大设计乘员重量：

Maximum design occupant weight:

### 100 kg

每次飞行前检查：

Before each flight, inspect:座椅结构；

Seat structure.座椅连接点

Seat attachment points.紧固件

Fasteners.

安全带及约束系统。 Restraint system.

发现碳纤维裂纹、分层或连接区域异常时，应停止使用并进行专项检查。 If carbon-fiber cracking, delamination, or abnormality at attachment

areas is found, the seat shall be removed from service pending detailed inspection.

# 第 12 章 定期维护计划

Chapter 12 Scheduled

Maintenance Program

### 项目 / Item	每次飞行	10 h	50 h	100 h	400 循环 500 h	800 循环

机体结构 / Airframe	●    ●    ●	●	—	●	—

碳纤维旋翼 / CF Rotors	●   ●   ●	●	—  定检	—电机 / Motors	●   —    ●	●	—    定检	—

ECS	●  —   ●	●	—  定检		—动力电池 / Propulsion Battery	●   ●   ●	●   定检	— 性能检查动力线束 / Power Wiring	●    ●    ●	●	—		●	—

信号线束 / Signal Wiring	●   ●   ●	●	—	●	—飞控 / Flight Control	●   —    ●	●	—	●	—

座椅 / Seat	●    ●    ●	●	—	●	—

救援降落伞 / Emergency Parachute

- —	—

按制造商

要求

按制造

—	—

商要求

### 注：500 小时定检是当前 SkyLo V1 的规定维护间隔；如后续试验、寿命分析或制造商技术通告要求缩短维护间隔，应以最新受控文件为准。

### Note: The 500-hour inspection interval is the current prescribed maintenance interval for the SkyLo V1. If subsequent testing, life

### assessment, or manufacturer technical instructions require a shorter interval, the latest controlled documentation shall take precedence.

第 13 章 时寿件与强制更换件清单 Chapter 13 Life-Limited Parts and Mandatory Replacement Items

## 总则

## General

SkyLo V1 部分安全关键部件具有规定的使用寿命、激活次数或维护期限。相关部件必须按照制造商技术文件、本维护手册及规定的维护周期进行检查、更换或维护。

Certain safety-critical components of the SkyLo V1 are subject to defined service lives, activation limits, or maintenance intervals. Such components shall be inspected, replaced, or maintained in accordance with the manufacturer's technical documentation, this Maintenance Manual, and the specified maintenance intervals.

当部件同时存在时间、飞行小时及使用次数等多项寿命限制时，应以首先达到的限制条件为准。

Where a component is subject to multiple life limits, including calendar

time, flight hours, or activation cycles, the applicable limit reached first shall apply.

## SkyLo V1 时寿件清单

## SkyLo V1 Life-Limited Parts List

序号	部件 / Component

寿命或维护限制 / Life or Maintenance Limit

维护要求 / Maintenance Requirement

GPS 底座 / GPS

1

Mount Base

2 年 / 2 years

到期更换 / Mandatory replacement

差分天线底座 /

2	Differential GNSS Antenna Mount Base

2 年 / 2 years

到期更换 / Mandatory replacement

GBS 10 救援降落伞系

3	统 / GBS 10 Rescue Parachute System

设计及测试约 7 次激活 / Approximately 7 activations design and test basis

按制造商手册管理 / Maintain per manufacturer manual

GBS 10 热释动器 /

4

GBS 10 Pyroactuator

保证 5 年或 100 飞行小时，以先到者为准 / 5 years or 100 flight hours, whichever occurs first

到期更换 / Replace upon limit

GBS 10 容器 / GBS 10 按系统维护要求 / Per system

5

每次检查，发现变形不得使

用 / Inspect each time;

Container

maintenance requirements

do not use if deformed

## GBS 10 救援降落伞系统

## GBS 10 Rescue Parachute System

SkyLo V1 采用 GBS 10 整机救援降落伞系统作为应急安全装置。

The SkyLo V1 is equipped with the GBS 10 whole-aircraft rescue parachute system as an emergency safety device.

GBS 10 系统的设计和测试寿命约为 7 次激活。

The GBS 10 system is designed and tested for approximately 7 activations.

该激活次数为系统设计及测试依据，不应在未经制造商确认的情况下将其简单理解为所有系统在第 7 次激活后均自动失效或必须报废。

This activation number represents the system design and test basis and shall not be interpreted, without confirmation from the manufacturer, as an automatic mandatory retirement of every system after the seventh activation.

救援降落伞系统必须严格按照制造商提供的使用和维护手册进行安装、检查、维护和操作。

The rescue parachute system shall be installed, inspected, maintained, and operated strictly in accordance with the manufacturer's operating and maintenance manual.

# GBS 10 容器检查

# GBS 10 Container Inspection

GBS 10 系统容器是救援系统的重要组成部分。

The GBS 10 container is a critical component of the rescue system.正确安装容器对于救援系统正常工作至关重要。

Correct installation of the container is essential for proper operation of the rescue system.

维护人员应重点检查：

Maintenance personnel shall pay particular attention to:容器是否变形；

Container deformation.容器是否存在裂纹； Cracks.

容器是否存在撞击损伤； Impact damage.

容器安装位置是否正确； Correct installation position.固定件是否松动

Loose attachment hardware.容器与周围结构是否存在干涉；

Interference between the container and surrounding structures.

### 特别要求：GBS 10 容器不得发生影响系统正常工作的变形。 Special requirement: The GBS 10 container shall not exhibit deformation that could affect proper system operation.

如果发现容器变形、严重撞击或其他可能影响救援系统工作的结构异常，应停止使用救援系统，并由授权人员按照 GBS 10 制造商维护文件进行检查和处置。

If container deformation, significant impact damage, or any other structural condition that could affect rescue-system operation is detected, the rescue system shall be removed from service pending inspection and disposition by authorized personnel in accordance with the GBS 10 manufacturer's maintenance documentation.

# GBS 10 热释动器

# GBS 10 Pyroactuator

GBS 10 热释动器（Pyroactuator）用于执行救援系统的启动动作，属于安全关键部件。

The GBS 10 Pyroactuator is used to initiate the rescue-system deployment sequence and is considered a safety-critical component.

根据制造商提供的信息，热释动器本身通常具有较长的储存寿命，其一般寿命可

达到至少 15 年。

According to the manufacturer's information, the Pyroactuator generally

has a long service life and may have a general lifespan of at least 15 years.

热释动器按照汽车行业相关规范及测试程序进行验证，包括：

The Pyroactuator is qualified in accordance with applicable automotive specifications and test programs, including:

- AKL-LV16

- USCAR-28

对于 SkyLo V1 系统，制造商目前提供的保证期限为：

For the SkyLo V1 system, the manufacturer's current warranty period is:

### 5 年。

### 5 years

因此，在达到上述任一限制条件后，应按照制造商维护要求更换热释动器。 Upon reaching either of the above limits, the Pyroactuator shall be replaced in accordance with the manufacturer's maintenance requirements.

### 维护注意 / Maintenance Note:

“至少 15 年”属于热释动器的一般寿命信息，不应替代 SkyLo V1

实际维护中采用的 5 年限制。对于航空器维护和放行，应采用制造商明确规定的限制值。

# GBS 10 激活后的维护

# Post-Activation Maintenance

GBS 10 系统一旦发生实际激活，不得未经检查直接恢复飞行使用。

Following an actual activation of the GBS 10 system, the aircraft shall not be returned to flight without the required inspection.

激活后必须对以下项目进行检查：

Following activation, the following items shall be inspected:救援降落伞；

Rescue parachute.

降落伞连接带；

Parachute attachment/suspension straps. GBS 10 容器；

GBS 10 container.热释动器； Pyroactuator.

发射/释放机构； Deployment/release mechanism.飞行器安装结构；

Aircraft mounting structure.与系统相关的电气连接；

Associated electrical connections.整机结构是否受到激活过程影响。

Aircraft structure for any effects resulting from activation.系统重新装机及恢复使用必须按照 GBS 10 制造商维护手册执行。

Reinstallation and return to service of the system shall be performed in

accordance with the GBS 10 manufacturer's maintenance manual.

# 救援系统维护记录

# Rescue System Maintenance Records

GBS 10 系统应建立独立的维护和激活记录。

A separate maintenance and activation record shall be maintained for the GBS 10 system.

记录至少包括：

Records shall include at least:

### 项目 / Item	记录内容 / Record

系统型号 / System Model	GBS 10

系统序列号 / System Serial Number	—

安装日期 / Installation Date	—热释动器序列号 / Pyroactuator Serial Number	—热释动器安装日期 / Pyroactuator Installation Date —当前激活次数 / Current Activation Count	—

上次检查日期 / Last Inspection Date	—热释动器更换日期 / Pyroactuator Replacement Date —容器检查结果 / Container Inspection Result	—降落伞检查结果 / Parachute Inspection Result	—维护人员 / Maintenance Personnel	—每次实际激活均应记录在案，并累计计算系统激活次数。

Every actual activation shall be recorded and included in the cumulative

activation count.

# GPS 底座维护要求

# GPS Mount Base Maintenance Requirements

GPS 底座属于飞行导航系统安装结构件，长期受到：

The GPS mount base is a structural mounting component for the navigation system and is subject to:

飞行振动； Flight vibration;温度循环；

Temperature cycling;紫外线及环境老化；

UV and environmental aging;结构疲劳载荷。

Structural fatigue loads.

维护要求：

Maintenance requirements:每次飞行检查：

Before each flight:

- 检查 GPS 底座固定状态；

- Check mounting security;

- 检查是否存在裂纹、松动或变形；

- Check for cracks, looseness, or deformation;

- 检查连接螺栓状态。

- Check fastener condition.

达到 2 年使用期限后：

Upon reaching two years of service:

- 必须更换 GPS 底座；

- Replace the GPS mount base;

- 重新检查安装精度；

- Verify installation accuracy;

- 重新进行导航系统功能测试。

- Perform navigation system functional verification.

# 差分天线底座维护要求

  - Differential GNSS Antenna

Mount Base Maintenance Requirements

差分天线底座用于保证 GNSS 差分天线安装位置稳定性，其结构状态直接影响定位精度。

The differential GNSS antenna mount base maintains antenna position stability and directly affects navigation accuracy.

维护要求：

Maintenance requirements:飞行前：

Before flight:

检查： Inspect:

- 天线底座固定状态；

- Antenna mount security;

- 天线安装角度；

- Antenna alignment;

- 底座裂纹及变形；

- Cracks or deformation;

- 连接紧固件。

- Fasteners.

达到 2 年期限后：

After two years of service:

必须更换差分天线底座。

The differential GNSS antenna mount base shall be replaced.更换后应完成：

After replacement:

- 天线安装检查；

- Antenna installation inspection;

- GNSS 定位功能检查；

- GNSS positioning verification;

- 差分定位精度检查。

- Differential positioning accuracy verification.

# 第 14 章 长期存放

# Chapter 14 Long-Term Storage

航空器超过 30 天不运行时，应执行长期存放程序。

When the aircraft will remain out of service for more than 30 days, the long-term storage procedure shall be performed.

主要要求：

Main requirements:

高压动力电池按规定储存 SOC 保存；

Store the high-voltage batteries at the specified storage SOC.断开动力系统；

Isolate the propulsion power system.防止雨水和潮气进入；

Protect against rain and moisture.防止碳纤维结构长期暴露于恶劣环境；

Protect carbon-fiber structures from prolonged exposure to adverse environments.

定期检查电池状态；

Periodically inspect battery condition.重新投入运行前完成完整维护检查。

Perform a complete maintenance inspection before returning the

aircraft to service.

# 第 15 章 维护记录

# Chapter 15 Maintenance Records

每项维护工作均应记录：

Every maintenance activity shall be recorded:航空器序列号 / Aircraft serial number 飞行小时 / Flight hours

日期 / Date

维护项目 / Maintenance item故障描述 / Defect description处理措施 / Corrective action

更换部件 / Replaced component

部件序列号 / Component serial number, when applicable维护人员 / Maintenance personnel

复检人员 / Inspector, when applicable

电机、ECS 及碳纤维旋翼的 500 小时定检必须单独形成维护记录。

The 500-hour inspections of motors, ECS units, and carbon-fiber rotors shall be separately documented.

# 第 16 章 维护放行

# Chapter 16 Return to Service

完成涉及以下系统的维护后，应进行相应功能检查：

After maintenance involving the following systems, the applicable functional checks shall be performed:

飞控系统 / Flight control system电机 / Motors

ECS

高压电池 / High-voltage batteries

高压配电 / High-voltage power distribution旋翼 / Rotors

主要结构 / Primary structure

救援降落伞系统 / Emergency parachute system

涉及关键飞行系统的维修完成后，未经规定的检查和必要验证，不得恢复飞行。 After maintenance of critical flight systems, the aircraft shall not be returned to flight until the required inspections and necessary verification have been completed.

# 附录 A：主要维护间隔 Appendix A: Principal Maintenance Intervals

### 部件 / Component	规定维护间隔 / Prescribed Interval

碳纤维旋翼 /

Carbon-Fiber Rotors

### 500 Flight Hours

电机 / Motors	500 Flight Hours

### ECS	500 Flight Hours

高压动力电池 / HV Batteries

按状态监控及电池维护程序 / Condition-based and battery maintenance program

飞控系统 / Flight Control System

按飞行小时及状态监控 / Flight-hour and

condition monitoring

主机身结构 / Primary Airframe

按定期检查程序 / Scheduled inspection program

救援降落伞 / Emergency Parachute

按制造商规定 / Per manufacturer requirements

附录 B：当前构型参数 Appendix B: Current Configuration Data

### 飞控软件版本 / Flight Control Software Version： V9 81.11.0

### PFD 软件版本：08.30

### 信号控制线缆 / Signal Control Cable：

PTFE 航空镀银屏蔽线

PTFE aerospace silver-plated shielded wire 4 × 0.35 mm²，OD 4.1 mm

Temperature range: −90°C to +260°C

### 主动力线缆 / Main Power Cable：

10 AWG / 6 mm² silicone cable持续载流能力：50 A

Continuous current rating: 50 A温度范围：−60°C to +200°C

### 最大成员重量 / Maximum Occupant Weight：

100 kg

### 电推进构型 / Propulsion Configuration：

四轴八桨分布式电推进

Four-axis, eight-rotor distributed electric propulsion动力电源架构 / Propulsion Power Architecture： 双路高压动力电池

Dual independent high-voltage propulsion battery systems电池 A → ECS → 上层旋翼

Battery A → ECS → Upper Rotors电池 B → ECS → 下层旋翼

Battery B → ECS → Lower Rotors

### PFD