[Excerpt from product manual Chapter 4]

4.2 Preflight Preparation

The pilot shall verify:

- Flight Manual has been reviewed

- He/she is physically fit for flight

- Is not under the influence of alcohol or drugs

- The weather suitable for VFR

- It is permissible to fly in the proposed area

- The battery is fully charged

- The flight is fully planned

4.3 Preflight Inspection & Checklist

4.3.1 Aircraft Structural Checks:

- Ensure no structural cracks

- Check cantilever arms are undamaged

- Ensure cantilever arms secure and rigid at joints

- Landing gear serviceable

- All critical fasteners marked with red torque-seal paint are secure and show no signs of loosening, fracture, or misalignment.

4.3.2 Propellers:

- Check all propellers are intact

- Ensure there are no cracks

- Check to ensure no deformation

- Make sure they are securely attached

- Check propeller tie wire fixing is intact

- Check all marked joints show no evidence of movement or misalignment.

4.3.3 Motors:

- Ensure Motors Rotate freely

- Check to ensure no excessive play

- Ensure there are no foreign objects

4.3.4 Battery System

- Secure front batter

- Secure rear battery

- Check to ensure no bulge in the battery

- Check for any physical damage

4.3.5 Ballistic Recovery System (BRS) / Parachute

- Safety pin removed

- System on standby

- Activation handle unobstructed

4.4 Power-Up Procedure

Main Power Connection

- Confirm both main power connectors are disconnected

- Connect Front Battery QS12 Main Power Connector

- Verify secure connection

- Connect Rear Battery QS12 Main Power Connector

- Verify secure connection

4.4.1 High Voltage System Safety Check - General

HayesX-250 utilizes a dual 108V DC electrical power system.

The front and rear battery packs are connected to the aircraft’s main power system through QS12 high-voltage connectors.

High-voltage system maintenance shall only be performed by trained personnel.

4.4.2 Preflight Inspection

Verify:

- QS12 connectors undamaged and in good condition

- No burn marks

- No thermal deformation

- No mechanical damage

- Locking mechanism is normal and functional

- High-voltage cable insulation intact

- No exposed conductors

4.4.3 Safety Warning

- Do not connect or disconnect QS12 connectors under electrical load.

- Do not short-circuit battery terminals.

- Do not operate with damaged connectors.

4.4.4 Abnormal Conditions

If any of the following occurs:

• Abnormal arcing

• Burning smell

• Connector overheating

• System startup failure

Disconnect power immediately and discontinue operation.

4.5 System Initialization

After both battery systems are connected, the Flight Control System powers up automatically.

- Flight Control System ON

- PFD ON

- Flight Data Recorder ON

- Wait for self-test completion

4.6 Propulsion System Check

Propulsion System Status Check:

Inspect all eight propulsion arm control modules:

- Eight green status indicators illuminated continuously

- No red fault indicators

- No warning tones

- No fault messages displayed

4.6.1 Motor Response Check

- Motor response normal

- No abnormal vibration

- No abnormal noise

4.7 Boarding Procedure

- Ensure aircraft is stable and secure before boarding

- Board aircraft

- Fasten restraint harness

- Check PFD display

- Ensure Battery Charge ≥95%

- No error warnings